A PROPOSAL TO DEVELOP SPMVV STUDENT SATELLITE

-Computer Science Role in the development

Kallu Nushrath  and Prof.V.Saritha

Email : nushrathkallu@gmail.com , vsaritha@spmvv.ac.in

Department of Computer Science and Engineering , School of Engineering and Technology,

Sri Padmavati Mahila Visvavidyalayam,Tirupati.

ABSTRACT:

The computer science role in student CubeSat development centers on the Onboard Data Handling (OBDH) system, software architecture, and ground support for commanding, data processing, and autonomous operations. The OBDH uses stackable PC/104 boards with radiation-tolerant processors (e.g., MSP430FR microcontrollers with ≥16 MHz clock, 32 kB flash) and interfaces like I2C/SPI/CAN for subsystem communication. Software creates a "flight app store" via inter-task message routers (e.g., Core Flight System architecture), handling telemetry output, command ingest, scheduling, and data storage (e.g., mass file systems for payload data). Roles include developing intuitive, open-source code for undergraduates, with plug-and-play capabilities for sensors and real-time monitoring. Attitude algorithms process GPS/magnetometer data for orientation, while AI enables tasks like cloud detection (PhiSat-1) or image compression. Ground stations use TLE sets for tracking and SDR for decoding. Student examples include KySat-2's Stellar Gyroscope software for rotation rate from image sequences, or CAPE-2's data handling for text-to-speech and file transfers. Development emphasizes tutorials, forums, and "like you fly" testing (e.g., Day In The Life simulations) to ensure no post-test code changes, focusing on low-power, fault-tolerant designs for missions like OPS-SAT with triple redundancy.